Conformance test specifications for

Wireless Access in
Vehicular Environments (WAVE) —
Security Services

Test Suite Structure and Test Purposes (TSS & TP)

| Document Mnemonics: | WAVE-16092-TSS&TP |
| --- | --- |
| Revision: | [V1.0] |
| Revision Date: | 3/23/2016 |

Table of Contents

1	Scope	4

2	References	4

2.1	Normative References	4

2.2	Informative References	4

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	General Convention	5

3.3	Abbreviations	5

4	Prerequisites and Test Configurations	5

4.1	Test Configurations	5

4.1.1	Global Test Parameters:	6

4.1.2	BSM Global Test Parameters	6

4.1.3	WSA Global Test Parameters	8

4.2	Feature Restriction and Pre-Enrolment	10

4.2.1	Feature Restriction	10

4.3	States in Initial Conditions	10

4.3.1	Conditions for the Initial State	11

5	Test Suite Structure (TSS)	12

5.1	Structure for security tests	12

5.2	Test groups	12

5.2.1	Root	12

5.2.2	Groups	12

5.2.3	Sub-Groups	12

5.2.4	Categories	12

6	Test Purposes (TP)	12

6.1	Introduction	12

6.1.1	TP definition conventions	12

6.1.2	TP Identifier naming conventions	13

6.1.3	Rules for the behaviour description	13

6.1.4	References	14

6.1.5	PICS selection and mnemonics for reference	14

6.1.6	Mnemonics for PICS reference	14

6.1.7	Sources of TP definitions	15

6.1.8	Basic Safety Messages	15

6.1.9	WAVE Service Advertisements	24

7	Messages and information element content	30

7.1	Basic Safety message (BSM):	30

7.1.1	Message defaults	30

7.1.2	Secure PDU Message Details	30

7.1.3	WSA Message Details	32

Appendix A:	35

Traceability Matrix	35

8	Revision History	44

# Scope

The scope of this document provides Test Suite Structure (TSS) and Test Purposes (TP’s) for WAVE Security Services as defined in IEEE 1609.2 [8]. Furthermore, the document defines a set of Test Purposes including Test Descriptions and the structure for the Test Suite.  The TP’s covers the Security Services requirements for BSM as specified SAE J2945/1 [1] and WSA as specified in IEEE 1609.3 [5]. The ISO standard for the methodology of conformance testing (ISO/IEC 9646-1 [3] and ISO/IEC 9646-2 [4]) as well as the ETSI rules for conformance testing (ETS 300 406 [7]) are used as a basis for the test methodology.

# References

## Normative References

The following referenced documents are necessary for the application of the present document.

[1]	SAE J2945/1 “Surface Vehicle Standard - On-board System Requirements for V2V Safety Communications”

[2]	IEEE Std. 1609.12-2016 “IEEE Standard for Wireless Access in Vehicular Environments – Identifier Allocations”

[3]	ISO/IEC 9646-1 (1994): "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework - Part 1: General concepts".

[4]	ISO/IEC 9646-2 (1994): "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework -- Part 2: Abstract Test Suite specification".

[5]	IEEE Std 1609.3-2016 “IEEE Standard for Wireless Access in Vehicular Environments (WAVE) — Network Services”.

[6]	ISO/IEC 9646-7 (1995): "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[7]	ETSI ETS 300 406 (1995): "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[8]	IEEE Std. 1609.2-2016: "IEEE Draft Standard for Wireless Access in Vehicular Environments - security Services for Applications and Management Messages".

## Informative References

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI EG 202 798 (V1.1.1): "Intelligent Transport Systems (ITS); Testing; Framework for conformance and interoperability testing".

# Definitions and abbreviations

## Definitions

For the purposes of the present document, the terms and definitions given in IEEE 1609.2 [8] , ISO/IEC 9646-1 [3] and in ISO/IEC 9646-7 [6] apply.

## General Convention

Parameters and its value defined in SAE J2945/1 [1], IEEE 1609.12 [2], IEEE 1609.3 [5] and IEEE 1609.2 [8] used in this document are donated as BOLD and ITALIC.

## Abbreviations

For the purposes of the present document, the following abbreviations apply:

BSM		Basic Safety Message

BI		Behaviour Invalid

BV		Behaviour Valid

CERTCH	Change Certificate

CA		Certificate Authority

EA		Enrolment Authority

ITS		Intelligent Transport Systems

IUT		Implementation Under Test

TC		Test Configuration System

TP		Test Purposes

TS		Test System

TSS		Test Suite Structure

PSID		Provider Service Identifier

PDU		Protocol Data Unit

SPDU		Secure Protocol Data Unit.

WAVE		Wireless Access in Vehicular Environments

WME		WAVE Management Entity

WSM		WAVE Short Message

WSA		WAVE Service Advertisement Message

SEND		Send message

SUT		System Under Test

RECV		Receive message

16092		Security Credentials

# Prerequisites and Test Configurations

## Test Configurations

This clause introduces the test configurations that is used to run the conformance testing for these definition of test purposes. These tests will be run in a lab environment in an automated fashion and controlled by the test system as shown in figure (1).. The test configurations cover the various scenarios of the IEEE 1609.2 test purposes.

WAVE

Host

IUT

TS

TS control interface

Figure 1: TC(1) Test Configuration System

### Global Test Parameters:

Default value parameters listed in this section will be used as a global test system parameters. These values are selected based on BSM and WSA relevant security profile as indicated in the reference column for each value.

### BSM Global Test Parameters

Below are listed global test parameters / conditions that are applicable to all BSM test cases in this specification.

#### Maximum certificate chain Length:

Select the default value for certificate according to the following table.

Table 4-1: certificate

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| certificate | Any positive integer | 1 | [1] Section 6.1.2.2.3 “ Table -11” |

#### Number of appPermissions included in the certificate:

Select the default value for appPermissions according to the following table.

Table 4-2: appPermissions

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| appPermissions | Any positive integer | 2 | [1] Section 6.1.2.1 “Table 6” |

#### Value for crlSeries Parameters:

Select the default values for crlSeries according to the following table.

Table 4-3:  CrlSeries

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| crlSeries | Unit16 - any positive integer value in the range of (0..65535)) | 1 | [8] section 5.1.3 |

#### Number of psid included in the certificate:

Select the default value for psid according to the following table.

Table 4-4: psid

| Parameter Name | Range of Values (p-encoded) | Default | Reference |
| --- | --- | --- | --- |
| psid | 1byte PSID: 0p00 to 0p7F | 0p20 “BSM” 0p26 “Misbehaviour for common applications.” | [2] Section “4.1.3“ Table 2 |
|  | 2byte PSID: 0p80-00 to 0pBF-FF |  |  |
|  | 3byte PSID: 0pC0-00-00 to 0pDF-FF-FF |  |  |
|  | 4byte PSID: 0pE0-00-00-00 to 0pEF-FF-FF-FF |  |  |

#### Number of identifiedRegion used for the certificate:

Select the default value for identifiedRegion according to the following table.

Table 4-5: identifiedRegion

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| identifiedRegion | Unit16 - any positive integer value in the range of (0..65535)) | 3 | [1] Section “6.1.2.2.4” Table 12 |

#### duration Life time Unit:

Select the default value for duration according to the following table.

Table 4-6: duration life time unit

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| duration | microseconds | hours | [8] Section “D.5.2.3” |
|  | milliseconds |  |  |
|  | seconds |  |  |
|  | minutes |  |  |
|  | hours |  |  |
|  | sixtyHours |  |  |
|  | years |  |  |

#### reconstructionValue:

Select the default value for reconstructionValue default value according to the following table.

Table 4-7: reconstructionValue

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| reconstructionValue | x-only fill compressed-y-0 compressed-y-1 uncompressed | compressed-y-0 | [8] Section “D.5.2.3” |

#### signature type:

Select the default value for signature according to the following table.

Table 4-8: signature

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| signature | ecdsaNistP256Signature ecdsaBrainpoolP256r1Signature | ecdsaNistP256Signature | [8] Section “5.3.1” |

#### “r” default value:

Select the default value for r according to the following table.

Table 4-9:  “r” value

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| r | x-only fill compressed-y-0 compressed-y-1 uncompressed | compressed-y-0 | [8] Section “D.5.2.3” |

#### Other Default values:

Select the default value for the parameter names listed on Table 4-10. The values for the parameter names listed on table 4-10 were obtained from

Table 4-10: default values

| Parameter Name | Value | Reference |
| --- | --- | --- |
| vMaxCertDigestInterval | 450 millisecond | [1] Section “7” Table 21 |
| +/-DE_DSecond/2 | 30 Seconds | [1] Section “6.1.2.2.3” Table 11 |
| vCertChangeInterval | 5 minutes | [1] Section “7” Table 21 |

### WSA Global Test Parameters

Below are listed global test parameters / conditions that are applicable to all WSA test cases in this specification.

#### Maximum certificate chain Length:

Select the default value for certificate according to the following table.

Table 4-11: certificate

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| certificate | Any positive integer | 1 | [5] Annex “H” Table H.1.1.3 |

#### id default value:

Select the default value for id according to the following table

Table 4-12: id

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| id | name | none | [8] Section “ 5.1.3” |
|  | binaryId |  |  |
|  | none |  |  |

#### Number of appPermissions included in the certificate:

Select the default value for appPermissions according to the following table.

Table 4-13: appPermissions

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| appPermissions | Any positive integer | 1 | [5] Annex “H” Table H.1.1.1 |

#### Value for cracaId & crlSeries Parameters:

Select the default values for cracaId & crlSeries according to the following table.

Table 4-14: cracaId & CrlSeries

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| cracaId | Octet String size(3) | 0 | [8] Section “5.1.1.3“ |
| crlSeries | Integer (0 . . 65535) | 0 |  |

#### duration Life time Unit:

Select the default value for duration according to the following table.

Table 4-15: duration life time unit

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| duration | microseconds | minutes | [8] |
|  | milliseconds |  |  |
|  | seconds |  |  |
|  | minutes |  |  |
|  | hours |  |  |
|  | sixtyHours |  |  |
|  | years |  |  |

#### Certificate region type:

Select the default value for region according to the following table.

Table 4-16: region

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| region | none | circularRegion | [5] Annex “H” Table H.1.1.4 |
|  | identified |  |  |
|  | circularRegion |  |  |

#### reconstructionValue:

Select the default value for reconstructionValue according to the following table.

Table 4-17: reconstructionValue

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| reconstructionValue | x-only | compressed-y-0 | [8] Section “D.5.2.3” |
|  | fill |  |  |
|  | compressed-y-0 |  |  |
|  | compressed-y-1 |  |  |
|  | uncompressed |  |  |

#### signature type:

Select the default value for signature according to the following table.

Table 4-18: signature

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| signature | ecdsaNistP256Signature | ecdsaNistP256Signature | [8] Section “5.3.1” |
|  | ecdsaBrainpoolP256r1Signature |  |  |

#### “r” default value:

Select the default value for r parameter according to the following table.

Table 4-19: r default value

| Parameter Name | Range of Values | Default | Reference |
| --- | --- | --- | --- |
| r | x-only | compressed-y-0 | [5] Annex “H” Table H.1.1.4 |
|  | fill |  |  |
|  | compressed-y-0 |  |  |
|  | compressed-y-1 |  |  |
|  | uncompressed |  |  |

## Feature Restriction and Pre-Enrolment

### Feature Restriction

In this clause all feature restrictions are listed:

- Encrypted PDUs are not considered

- Decrypting encrypted SPDUs are not considered.

- Peer to peer certificate distribution (P2PCD) is not considered

- Service Access Points (SAPs) are not considered.

- Certificate Revocation List (CRL) Verification Entity is not considered.

## States in Initial Conditions

The description of the TP is built according to EG 202 798 [i.1].

Test purposes use a generic "Initial State" that corresponds to a state where the IUT is ready for starting the test execution. Furthermore, the IUT shall be left in this "Initial State", when the test is completed.

Being in the "Initial State" refers to the starting point of the initial device configuration. There are no pending actions, no instantiated buffers or variables, which could disturb the execution of a test.

### Conditions for the Initial State

Figure 2 depicts the overall state diagram for a test system below.

State 2

IUT is in “Initial State”

State 1

IUT is powered off

State 3

Test Purpose Initial Conditions/Pre-test Conditions

State 4

Test Execution

Figure 2: State Diagram

Each TP contains an initial condition. The initial condition defines the initial state in which the IUT has to be to apply the actual TP. Most of the TPs start from the “initial state” which is defined as follows:

- The IUT is powered up.

- The IUT is not transmitting or receiving messages

- The IUT is provisioned with the appropriate security credentials to enable transmission or reception of messages. That is, the IUT is configured with a valid signer credentials (certificate) as specified in SAE J2945/1 [1] and IEEE 1609.3 [5] security profiles for BSM and WSA.

Some TPs start from a different initial condition which is explicitly defined in the TP such as if an invalid behavior needs to be tested by the IUT. However, the “initial state” defined above is the starting point before the different initial conditions are established.

When the execution of the initial condition does not succeed, it leads to the assignment of an Inconclusive verdict.

# Test Suite Structure (TSS)

## Structure for security tests

The test suite is structured as a tree with the root defined as 16092. The tree is of rank 4 with the first rank is Root, 16092 second is Group, third is Sub-group and the fourth rank is the standard ISO conformance test categories. The Sub-Group (third rank) belongs to any Group member in the second rank.

## Test groups

The test suite has a total of four levels. The first level is the root. The second level separates the root into various functional areas. The third level is the sub-functional areas if necessary. The fourth level is the standard ISO conformance test categories.

### Root

The root identifies the 1609.2 protocol given in IEEE 1609.2 [8].

### Groups

This level contains two message types identified as:

BSM

WSA

### Sub-Groups

This level contains functional areas identified in Table 5-1.

Table 5-1: Functional areas

| Functional areas | Description |
| --- | --- |
| Send/Transmit | The IUT signs and transmit WSM |
| Receive | The IUT receive and verifies WSM |
| Change Certificate | The IUT changes the signing certificate for BSM as per 2945/1 requirement |

### Categories

This level contains the standard ISO conformance test categories limited to the behaviour valid event and behaviour invalid event.

# Test Purposes (TP)

## Introduction

### TP definition conventions

A Test Purpose (TP) is a prose description of a well-defined objective of testing. Applying to conformance testing, it focuses on a single conformance requirement or a set of related conformance requirements from the base standards [i.1]..The TP definition is built according to EG 202 798 [i.1].

The TPs are defined by the rules shown in Table 6-1.

Table 6-1: TP definition rules

| Test Purpose ID | The Test Purpose ID is a unique identifier. It shall be specified according to the TP naming conventions defined in the clause below. |
| --- | --- |
| Test objective | Short description of test purpose objective according to the requirements from the base standard. |
| References | The reference indicates the sub-clauses of the reference standard specifications in which the conformance requirement is expressed. |
| Test Configuration | The Config Id references the test configuration selected for this TP. |
| PICS Selection | Reference to the PICS statement involved for selection of the TP. It may contain a Boolean expression. |
| Pre-Test Conditions | A list of test specific pre-conditions that need to be met by the SUT including information about equipment configuration, i.e. precise description of the initial state of the SUT required to start executing the test sequence |
| Test Sequence | An ordered list of equipment operation and observations. In case of a conformance test description the test sequence contains also the conformance checks as part of the observations |
| Event Types |  |
| Stimulus | Corresponds to an event that enforces an IUT to proceed with a specific protocol action, like sending a message for instance. |
| Check | Ensures the receipt of protocol messages on reference points with valid content. |
| Verify | Consists of verifying that the IUT behaves according to the expected behavior (for instance the IUT behavior shows that it receives the expected message). |
| Configure | Corresponds to an action to modify the IUT configuration. |

When a conformance test has a sequencing requirement, these are described using a format in the table 3 derived from [i.1]

### TP Identifier naming conventions

The identifier of the TP is built according to Table 6-2.

Table 6-2:TP naming convention

| Identifier | TP-<root>-<gr>-<sgr>-<x>-<nn> or TP-<root>-<gr>-<x>-<nn> when no <sgr> |  |  |
| --- | --- | --- | --- |
|  | <root> = root | 16092 | 1609.2 |
|  | <gr> = group | BSM | Basic Safety Message |
|  |  | WSA | Wave Service Advertisement message |
|  | <sgr> =sub- group | SEND | Send Message |
|  |  | RECV | Receive Message |
|  |  | CERTCH | Change Certificate |
|  | <x> = type of testing | BV | Valid Behaviour tests |
|  |  | BI | Invalid Syntax or Behaviour Tests |
|  | <nn> = sequential number |  | 01 to 99 |

### Rules for the behaviour description

The description of the TP is built according to EG 202 798 [i.1].

The base standards are not using finite state machine concept. As consequence, the test purposes use a generic "Initial State" that corresponds to a state where the IUT is ready for starting the test execution. Furthermore, the IUT shall be left in this "Initial State", when the test is completed.

Being in the "Initial State" refers to the starting point of the initial device configuration. There are no pending actions, no instantiated buffers or variables, which could disturb the execution of a test.

### References

All Test Purposes are derived from requirements defined in 1609.2 [8]. Traceability between TPs and sub-clauses of referenced standard specifications is established in Table A- 1 for BSM and Table A-2 for WSA messages. For each PICS, a reference section from 1609.2 [8] is listed and applicable test purposes are identified in the TP ID column.

### PICS selection and mnemonics for reference

Table A- 1 and Table A- 2 includes a subset of PICS defined in 1609.2 [8] with a traceability to TPs included in the TP ID column. Some TPs are directly derived from SAE J2945/1[1] requirements and do not refer to any PICS from 1609.2[8]. In this case the SAE J2945/1[1] requirement that is used to generate the test purpose is listed in the “Reference section” of the TP.

Table 6-3 lists mnemonic names and maps them to a subset of PICS item number. This is a partial list of PICS used in selecting of certain TPs or TPs which incorporated variances.

### Mnemonics for PICS reference

The following table lists mnemonic names and maps them to the PICS item number.  This is a partial list of PICS used in selecting TPs. The complete list of PICS with traceability to TPs is included in Appendix A.

Table 6-3: Mnemonics for PICS reference

| Mnemonic | PICS item |
| --- | --- |
| PIC_Generate_SignedData | [8] Annex A, S1.2.2 |
| PIC_Generate_Using_Valid_HashAlgorithm | [8] Annex A, S1.2.2.1 |
| PIC_Generate_Signing_With_SHA256 SHA-256 | [8] Annex A, S1.2.2.1.1 |
| PIC_Generate_Signed_Data_payload | [8] Annex A, S1.2.2.2 |
| PIC_Generate_With_Payload_Containing_Data | [8] Annex A, S1.2.2.2.1 |
| PIC_Generate_With_generationTime_In_security_headers headers | [8] Annex A, S1.2.2.2.3 |
| PIC_Generate_With_generationLocation_In_security_headers 16092urity_Headers | [8] Annex A, S1.2.2.2.5 |
| PIC_Generate_Support_SignerIdentifier | [8] Annex A, S1.2.2.3 |
| PIC_Generate_Of_Type_digest | [8] Annex A, S1.2.2.3.1 |
| PIC_Generate_Of_Type_certificate | [8] Annex A, S1.2.2.3.2 |
| PIC_Generate_Max_Number_Of_Certificates_In_The_chain | [8] Annex A, S1.2.2.3.2.1 |
| PIC_Generate_Signature | [8] Annex A, S1.2.2.4 |
| PIC_Generate_Ecdsa256_Signature | [8] Annex A, S1.2.2.4.1 |
| PIC_Generate_Ecdsa256_Signature_Using_ NIST p256 | [8] Annex A, S1.2.2.4.1.1 |
| PIC_Generate_Signature_With_Compressed_r_value | [8] Annex A, S1.2.2.4.1.5 |
| PIC_Generate_Support_signing_Implicit_Certificate | [8] Annex A, S1.2.2.8 |
| PIC_Verify_Ieee1609DoT2Data_Containing_SignedData | [8] Annex A, S1.3.2 |
| PIC_Verify_Using_Valid_HashAlgorithm | [8] Annex A, S1.3.2.1 |
| PIC_Verify_Signing_With_SHA256 | [8] Annex A, S1.3.2.1.1 |
| PIC_Verify_Signed_Data_payload | [8] Annex A, S1.3.2.2 |
| PIC_Verify_With_Payload_Containing_Data | [8] Annex A, S1.3.2.2.1 |
| PIC_Verify_With_generationTime_In_security_headers | [8] Annex A, S1.3.2.2.3 |
| PIC_Verify_With_generationLocation_In_security_headers | [8] Annex A, S1.3.2.2.5 |
| PIC_Verify_Support_SignerIdentifier | [8] Annex A, S1.3.2.3 |
| PIC_Verify_Of_Type_digest | [8] Annex A, S1.3.2.3.1 |
| PIC_Verify_Of_Type_certificate | [8] Annex A, S1.3.2.3.2 |
| PIC_Verify_Max_Number_Of_Certificates_In_The_chain | [8] Annex A, S1.3.2.3.2.1 |
| PIC_Verify_Signature | [8] Annex A, S1.3.2.4 |
| PIC_Verify_ecdsa256_Signature | [8] Annex A, S1.3.2.4.1 |
| PIC_Verify_ecdsa256_Signature_Using_ NIST_p256 | [8] Annex A, S1.3.2.4.1.1 |
| PIC_Verify_Signature_With_Compressed_r_value | [8] Annex A, S1.3.2.4.1.4 |
| PIC_Verify_SignedData_fails_if_certificate_is_not_valid | [8] Annex A, S1.3.2.5 |
| PIC _Verify_Reject_data if_certificate_doesn’t_have_proper_appPermissions | [8] Annex A, S1.3.2.5.2 |
| PIC_Verify_Reject_data_if_generationTime_not_available | [8] Annex A,S 1.3.2.10.4 |
| PIC_Verify_Reject_data_if_generationLocation_not_available | [8] Annex A, S1.3.2.10.5 |

### Sources of TP definitions

All TPs are specified according to IEEE 1609.2 [8] and SAE J2945/1 [1]. Test purposes for 1609.2

### Basic Safety Messages

#### Transmission of packets

| Identifier |  | TP-16092-BSM-SEND-BV-01 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will generate a valid BSM security header. Security header shall include, protocolVersion, content, signedData, hashId, tbsData, headerInfo and doesn’t include expiryTime nor generationLocation. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit more than one BSM per second as defined in Table 7-1 |  |
| 2 | Stimulus | The IUT transmits BSMs |  |
| 3 | Verify | BSM Ieee1609Dot2Data contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 4 | Verify | BSM Ieee1609Dot2Data contains content indicating signedData | Pass/Fail |
| 5 | Verify | BSM signedData contains hashId indicating sha256 | Pass/Fail |
| 6 | Verify | BSM tbsData contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 7 | Verify | BSM tbsData contains content indicating unsecuredData (Payload Data> 0) | Pass/Fail |
| 8 | Verify | BSM headerInfo contains psid indicating value = 0p20 | Pass/Fail |
| 9 | Verify | BSM headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) | Pass/Fail |
| 10 | Verify | BSM headerInfo doesn’t include expiryTime | Pass/Fail |
| 11 | Verify | BSM headerInfo doesn’t include generationLocation | Pass/Fail |

| Identifier |  | TP-16092-BSM-SEND-BV-02 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the BSM digitally signed by certificate contains a valid 1609.2 certificate data structure. The certificate shall include a valid signer info, toBeSigned linkageData information, valid region information and ecdsaP256Signature type. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit more than one BSM per second as defined in Table 7-3 |  |
| 2 | Stimulus | The IUT transmits BSMs |  |
| 4 | Verify | BSMs signer contains certificate indicating value=0x01 | Pass/Fail |
| 5 | Verify | BSM signer contains version indicating value = 0x03 | Pass/Fail |
| 6 | Verify | BSM signer contains type indicating implicit | Pass/Fail |
| 7 | Verify | BSM signer contains issuer containing sha256AndDigest indicating HashedId8 (a non-zero value of size 8 octets) | Pass/Fail |
| 8 | Verify | BSM toBeSigned contains id indicating linkageData | Pass/Fail |
| 9 | Verify | BSM linkageData contains iCert indicating a value of size 2 octets | Pass/Fail |
| 10 | Verify | BSM linkageData contains linkage-value indicating value of size 9 octets | Pass/Fail |
| 11 | Verify | BSM linkageData contains group-linkage-value containing jValue indicating a value of size 4 octets | Pass/Fail |
| 12 | Verify | BSM linkageData contains group-linkage-value containing value indicating a value of size 9 octets | Pass/Fail |
| 13 | Verify | BSM toBeSigned contains cracaId indicating a non-zero value of size 3 octets | Pass/Fail |
| 14 | Verify | BSM toBeSigned contains crlSeries indicating a value =0x01 | Pass/Fail |
| 15 | Verify | BSM toBeSigned contains start indicating Time32 (a non-zero value of size 4 octets) | Pass/Fail |
| 16 | Verify | BSM toBeSigned contains duration containing hours indicating Unit16 (a non-zero Integer value of size 2 octets) | Pass/Fail |
| 17 | Verify | BSM toBeSigned contains region containing identifiedRegion indicating value=0x03 | Pass/Fail |
| 18 | Verify | BSM toBeSigned contains identifiedRegion1 containing countryOnly indicating value=0x7C | Pass/Fail |
| 19 | Verify | BSM toBeSigned contains identifiedRegion2 containing countryOnly indicating value=0x1E4 | Pass/Fail |
| 20 | Verify | BSM toBeSigned contains identifiedRegion3 containing countryOnly indicating value=0x348 | Pass/Fail |
| 21 | Verify | BSM toBeSigned contains appPermission indicating value=0x02 | Pass/Fail |
| 22 | Verify | BSM toBeSigned contains psid indicating a value=0p20 | Pass/Fail |
| 23 | Verify | BSM toBeSigned contains Psid indicating value=0p26 | Pass/Fail |
| 24 | Verify | BSM toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating compressed-y-0 (value of size 32 octets) | Pass/Fail |
| 25 | Verify | BSM signature contains ecdsaP256Signature indicating r (compressed-y-0 consists of octet size 32 ) | Pass/Fail |
| 26 | Verify | BSM signature contains opaque s indicating non-zero value of size 32 octets | Pass/Fail |

| Identifier |  | TP-16092-BSM-SEND-BV-03 |  |  |
| --- | --- | --- | --- | --- |
| Summary |  | Validate that the BSM signed by certificate digest contains a valid 1609.2 data structure. The BSM shall include, protocolVersion, content, signedData, hashId, tbsData, headerInfo, signer, ecdsaP256Signature and doesn’t include expiryTime nor generationLocation. |  |  |
| Test Configuration |  | TC(1) |  |  |
| IUT |  | IUT |  |  |
| Reference: |  |  |  |  |
| PICS Selection |  |  |  |  |
| Pre-test conditions |  |  |  |  |
| - The IUT is being initialized |  |  |  |  |
| Test Sequence |  |  |  |  |
| Step | Type |  | Description | Verdict |
| 1 | Configure |  | The IUT is configured to transmit more than one BSM per second as defined in Table 7-2 |  |
| 2 | Stimulus |  | The IUT transmits BSMs |  |
| 3 | Verify |  | BSM Ieee1609Dot2Data contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 4 | Verify |  | BSM Ieee1609Dot2Data contains content indicating signedData | Pass/Fail |
| 5 | Verify |  | BSM signedData contains hashId indicating sha256 | Pass/Fail |
| 6 | Verify |  | BSM tbsData contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 7 | Verify |  | BSM tbsData contains content indicating unsecuredData (Payload Data> 0) | Pass/Fail |
| 8 | Verify |  | BSM headerInfo contains psid indicating value = 0p20 | Pass/Fail |
| 9 | Verify |  | BSM headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) | Pass/Fail |
| 10 | Verify |  | BSM headerInfo doesn’t include expiryTime | Pass/Fail |
| 11 | Verify |  | BSM headerInfo doesn’t include generationLocation | Pass/Fail |
| 12 | Verify |  | BSMs contains signer containing digest indicating HashedId8 (a non-zero value of size 8 octets) | Pass/Fail |
| 13 | Verify |  | BSM signature contains ecdsaP256Signature indicating r (compressed-y-0 consists of octet size 32 ) | Pass/Fail |
| 14 | Verify |  | BSM signature contains opaque s indicating non-zero value of size 32 octets | Pass/Fail |

| Identifier |  | TP-16092-BSM-SEND-BV-04 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the BSM is digitally signed by certificate at least every vMaxCerDigestInterval. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  | SAE J2945 [1] Table 10 “Security Profile for Transmitting BSMs”. |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit more than one BSM per 450 ms as defined in Table 7-3 |  |
| 2 | Stimulus | The IUT transmits BSMs |  |
| 3 | Verify | IUT transmitted BSM at TIME_1 contains signer indicating certificate where the low order 8 octets of the sha256 hash is calculated for the Certificate (ID1) | Pass/Fail |
| 4 | Verify | IUT transmitted the next BSM at TIME_2 (TIME_2>TIME_1) | Pass/Fail |
| 5 | Verify | IUT transmitted at TIME_2 contains signer indicating certificate where the low order 8 octets of the sha256 hash is calculated for the Certificate (ID2) | Pass/Fail |
| 6 | Verify | ID2 != ID1 | Pass/Fail |
| 7 | Verify | (TIME_2 - TIME_1) ' greater than or equal to' vMaxCerDigestInterval | Pass/Fail |

| Identifier |  | TP-16092- BSM-SEND-BV-05 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that a BSM containing a certificate digest is signed using a valid digital signature computed over entire payload using ecdsaP256Signature type. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit more than one BSM per second |  |
| 2 | Stimulus | The IUT transmits BSM |  |
| 3 | Verify | The IUT transmitted BSMs contains signer containing digest indicating HashedId8 where HashedId8 is referenced to pre-loaded certificate on the IUT and containing verificationkeyIndicator (KEY) | Pass/Fail |
| 4 | Verify | BSM Signature contains ecdsaP256Signature indicating r and s values verifiable using KEY. | Pass/Fail |

| Identifier |  | TP-16092-BSM-SEND-BV-06 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that a BSM digitally signed by certificate contains a valid signature computed over entire payload using ecdsaP256Signature type. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit more than one BSM per second as defined in Table 7-3 |  |
| 2 | Stimulus | The IUT transmits BSM |  |
| 3 | Verify | BSM signer contains certificate indicating value=0x01 | Pass/Fail |
| 4 | Verify | BSM signer contains type indicating implicit | Pass/Fail |
| 5 | Verify | BSM toBeSigned contains psid indicating a value=0p20 | Pass/Fail |
| 6 | Verify | BSM toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating compressed-y-0 (value of size 32 octets) (KEY) | Pass/Fail |
| 7 | Verify | BSM signature contains ecdsaP256Signature indicating r and s values verifiable using (KEY) | Pass/Fail |

#### Reception of packets

| Identifier |  | TP-16092-BSM-RECV-BV-01 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credentials for a well-formed BSM security header. Security header shall include protocolVersion, signedData, tbsData, headerInfo and doesn’t include expiryTime nor generationLocation. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second |  |
| 2 | Check | BSM Ieee1609Dot2Data contains protocolVersion indicating (value = 0x03) |  |
| 3 | Check | BSM Ieee1609Dot2Data contains content indicating signedData |  |
| 4 | Check | BSM signedData contains hashId indicating sha256 |  |
| 5 | Check | BSM tbsData contains protocolVersion indicating value = 0x03 |  |
| 6 | Check | BSM tbsData contains content indicating unsecuredData (Payload Data> 0) |  |
| 7 | Check | BSM headerInfo contains psid indicating value = 0p20 |  |
| 8 | Check | BSM headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) |  |
| 9 | Check | BSM headerInfo doesn’t include expiryTime |  |
| 10 | Check | BSM headerInfo doesn’t include generationLocation |  |
| 11 | Stimulate | The IUT receives BSMs |  |
| 12 | Verify | IUT indicate that the security header for BSM is formed correctly | Pass/Fail |

| Identifier |  | TP-16092-BSM-RECV-BV-02 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credential for a well-formed BSM signed by implicit certificate. The BSM shall include protocolVersion, signedData, tbsData, headerInfo, signer, toBeSigned, linkageData, ecdsaP256Signature type and doesn’t include expiryTime nor generationLocation. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second as defined in Table 7-3 |  |
| 2 | Check | BSM signer contains certificate indicating value= 0x01 |  |
| 3 | Check | BSM signer contains version indicating value = 0x03 |  |
| 4 | Check | BSM signer contains type indicating implicit |  |
| 5 | Check | BSM signer contains issuer containing sha256AndDigest indicating HashedId8 a non-zero value of size 8 octets |  |
| 6 | Check | BSM toBeSigned contains id indicating linkageData |  |
| 7 | Check | BSM linkageData contains iCert indicating a value of size 2 octets |  |
| 8 | Check | BSM linkageData contains linkage-value indicating value of size 9 octets |  |
| 9 | Check | BSM linkageData contains group-linkage-value containing jValue indicating a value of size 4 octets |  |
| 10 | Check | BSM linkageData contains group-linkage-value containing value indicating a value of size 9 octets |  |
| 11 | Check | BSM toBeSigned contains cracaId indicating a non-zero value of size 3 octets |  |
| 12 | Check | BSM toBeSigned contains crlSeries indicating a value =0x01 |  |
| 13 | Check | BSM toBeSigned contains start indicating Time32 (a non-zero value of size 4 octets) |  |
| 14 | Check | BSM toBeSigned contains duration containing hours indicating Unit16 (a non-zero Integer value of size 2 octets) |  |
| 15 | Check | BSM toBeSigned contains region containing identifiedRegion indicating value=0x03 |  |
| 16 | Check | BSM toBeSigned contains identifiedRegion1 containing countryOnly indicating value=0x7C |  |
| 17 | Check | BSM toBeSigned contains identifiedRegion2 containing countryOnly value=0x1E4 |  |
| 18 | Check | BSM toBeSigned contains identifiedRegion3 containing countryOnly value=0x348 |  |
| 19 | Check | BSM toBeSigned contains appPermission indicating value=0x2 |  |
| 20 | Check | BSM toBeSigned contains psid indicating a value=0p20 |  |
| 21 | Check | BSM toBeSigned contains Psid indicating a value=0p26 |  |
| 22 | Check | BSM toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating compressed-y-0 (value of size 32 octets) |  |
| 23 | Check | BSM signature contains ecdsaP256Signature indicating r (compressed-y-0 consists of octet size 32 ) |  |
| 24 | Check | BSM signature contains opaque s indicating non-zero value of size 32 octets |  |
| 25 | Stimulate | The IUT receives BSMs. |  |
| 26 | Verify | IUT indicates that the BSM holds a valid security credentials. | Pass/Fail |

| Identifier |  | TP-16092-BSM-RECV-BV-03 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credential for a well-formed BSM signed by certificate digest of known certificate. The BSM shall include, protocolVersion, content, signedData, tbsData, headerInfo, signer, ecdsaP256Signature type and doesn’t include expiryTime nor generationLocation. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second as defined in Table 7-2 |  |
| 2 | Check | BSM Ieee1609Dot2Data contains protocolVersion indicating (value = 0x03) |  |
| 3 | Check | BSM Ieee1609Dot2Data contains content indicating signedData |  |
| 4 | Check | BSM signedData contains hashId indicating sha256 |  |
| 5 | Check | BSM tbsData contains protocolVersion indicating value = 0x03 |  |
| 6 | Check | BSM tbsData contains content indicating unsecuredData (Payload Data> 0) |  |
| 7 | Check | BSM headerInfo contains psid indicating value = 0p20 |  |
| 8 | Check | BSM headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) |  |
| 9 | Check | BSM headerInfo doesn’t include expiryTime |  |
| 10 | Check | BSM headerInfo doesn’t include generationLocation |  |
| 11 |  | BSMs contains signer containing digest indicating HashedId8 (a non-zero value of size 8 octets) |  |
| 12 | Check | BSM signature contains ecdsaP256Signature indicating r (compressed-y-0 consists of octet size 32 ) |  |
| 13 | Check | BSM signature contains opaque s indicating non-zero value of size 32 octets |  |
| 14 | Stimulate | IUT receives BSMs |  |
| 15 | Verify | IUT indicates that the BSM holds a valid security credentials. | Pass/Fail |

| Identifier |  | TP-16092-BSM-RECV-BV-04 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credential for a BSM digitally signed by certificate, which includes generationTime within +/-DE_DSecond/2 of the current time and the BSM generationTime is earlier than the expiration time of the signing certificate. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  | SAE J2945 [1] Table 11 “Security Profile for Receiving BSMs” |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second as defined in Table 7-3 |  |
| 2 | Check | BSM headerInfo contains psid indicating value = 0p20 |  |
| 3 | Check | BSM headerInfo contains generationTime indicating a TIME_1 where (CUR_TIME - DE_DSecond/2 'less or equal' TIME_1 'less or equal' CUR_TIME + DE_DSecond/2) |  |
| 4 | Check | BSMs signer contains certificate indicating value=0x01 |  |
| 5 | Check | BSM signer contains type indicating implicit |  |
| 6 | Check | BSM toBeSigned contains start & duration indicating EXP_TIME where (CUR_TIME 'less or equal' EXP_TIME) |  |
| 7 | Stimulate | The IUT receives BSMs. |  |
| 8 | Verify | IUT indicates that the BSM holds a valid security credentials. | Pass/Fail |

| Identifier |  | TP-16092-BSM-RECV-BV-05 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credential for a BSM digitally signed by certificate digest which includes generationTime within +/-DE_DSecond/2 from the current time, and the BSM is generated before the expiration time of the signing certificate digest pre-stored on the device. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  | SAE J2945 [1] Table 11 “Security Profile for Receiving BSMs” |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second as defined in Table 7-2 |  |
| 2 | Check | BSM headerInfo contains psid indicating value = 0p20 |  |
| 3 | Check | BSM contains signer containing digest indicating HashedId8 (ID1) |  |
| 4 | Stimulate | The IUT receives BSMs |  |
| 5 | Check | BSM headerInfo contains generationTime indicating TIME_1 where (CUR_TIME – DE_DSecond/2 'less or equal' TIME_1 'less or equal' CUR_TIME + DE_DSecond/2 ) |  |
| 6 | Check | BSM contains signer containing digest indicating HashedId8 (ID1) |  |
| 7 | Check | BSM toBeSigned contains start & duration indicating EXP_TIME where (CUR_TIME 'less or equal' EXP_TIME) |  |
| 8 | Stimulate | The IUT receives BSMs |  |
| 7 | Verify | IUT indicates that the BSM holds a valid security credentials. | Pass/Fail |

#### Certificate Rotation Validation

| Identifier |  | TP-16092-BSM-CERTCHG-BV-01 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the BSM contains either certificate or certificate digest referencing the same certificate for (vCertChangeInterval) minutes and BSM changes the referenced certificate after (vCertChangeInterval). |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  | SAE J2945 [1] section 6.3.5 “6.5.3-V2V-SECPRIV-CERTCHG-001” |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized - Time is set at the moment when digest changes |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit more than one BSM per second |  |
| 2 | Stimulus | The IUT transmits BSM at TIME_1 |  |
| 3 | Verify | BSMs contains signer containing digest indicating HashedId8 (ID1) if yes go to step 5 | Pass/Fail |
| 4 | Verify | BSM signer contains certificate indicating value= 0x01 where the low order 8 octets of the sha-256 hash is calculated for the certificate (ID1) | Pass/Fail |
| 5 | Verify | The IUT sends the next BSM at TIME_2 where (TIME_2-TIME_1) 'less' 1sec | Pass/Fail |
| 6 | Verify | BSMs contains signer containing digest indicating HashedId8 (ID2) if yes go to step 8 | Pass/Fail |
| 7 | Verify | BSM signer contains certificate indicating value=0x01 where the low order 8 octets of the sha-256 hash is calculated for the certificate (ID2) | Pass/Fail |
| 8 | Verify | where ID2 = ID1 | Pass/Fail |
| 9 | Verify | IUT sends BSM at TIME_3 | Pass/Fail |
| 10 | Verify | BSMs contains signer containing digest indicating HashedId8 (ID3) if yes go to step 12 | Pass/Fail |
| 11 | Verify | BSM signer contains certificate indicating value= 0x01 where the low order 8 octets of the sha-256 hash is calculated for the certificate (ID3) | Pass/Fail |
| 12 | Verify | where ID3 != ID2 | Pass/Fail |
| 13 | Verify | vCertChangeInterval 'less or equal' (TIME_3 - TIME_2) 'less or equal' vCertChangeInterval+ 30 sec | Pass/Fail |

#### Reception of packets – invalid behaviour tests

| Identifier |  | TP-16092-BSM-RECV-BI-01 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate an invalid security credentials for a BSM signed by certificate digest, which failed verification due to incorrect signature. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second |  |
| 2 | Check | BSM headerInfo contains psid indicating value = 0p20 |  |
| 3 | Check | BSMs contains signer containing digest indicating HashedId8 (a non-zero value of size 8 octets) |  |
| 4 | Check | BSM signature contains ecdsaP256Signature type indicating r and s signature not verifiable using KEY |  |
| 5 | Stimulate | The IUT receives BSMs |  |
| 6 | Verify | IUT indicates that the BSM holds an invalid security credentials | Pass/Fail |

| Identifier |  | TP-16092- BSM-RECV-BI-02 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate an invalid BSM signed by implicit certificate which failed verification due to incorrect signature. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one BSM per second |  |
| 2 | Check | BSMs signer contains certificate indicating value=0x01 |  |
| 3 | Check | BSM signer contains type indicating implicit |  |
| 4 | Check | BSM toBeSigned contains psid indicating a value=0p20 |  |
| 5 | Check | BSM toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating compressed-y-0 (value of size 32 octets) (KEY) |  |
| 6 | Check | BSM signature contains ecdsaP256Signature type indicating r and s signature not verifiable using KEY |  |
| 7 | Stimulate | The IUT receives BSMs |  |
| 8 | Verify | IUT indicates that the BSM holds an invalid security credentials | Pass/Fail |

### WAVE Service Advertisements

#### Transmission of packets

| Identifier |  | TP-16092-WSA-SEND-BV-01 |  |  |
| --- | --- | --- | --- | --- |
| Summary |  | Validate that the IUT will generate a correct 1609.2 WSA security header structure. That is, the WSA message shall include protocolVersion, content, signedData, tbsData and headerInfo. |  |  |
| Test Configuration |  | TC(1) |  |  |
| IUT |  | IUT |  |  |
| Reference: |  |  |  |  |
| PICS Selection |  |  |  |  |
| Pre-test conditions |  |  |  |  |
| - The IUT is being initialized |  |  |  |  |
| Test Sequence |  |  |  |  |
| Step | Type |  | Description | Verdict |
| 1 | Configure |  | The IUT is configured to transmit one or more WSA per second as defined in Table 7-5 |  |
| 2 | Stimulus |  | The IUT transmits WSAs |  |
| 3 | Verify |  | WSA Ieee1609Dot2Data contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 4 | Verify |  | WSA Ieee1609Dot2Data contains content indicating signedData | Pass/Fail |
| 5 | Verify |  | WSA signedData contains hashId indicating sha256 | Pass/Fail |
| 6 | Verify |  | WSA tbsData contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 7 | Verify |  | WSA tbsData contains content indicating unsecuredData (Payload Data> 0) | Pass/Fail |
| 8 | Verify |  | WSA headerInfo contains psid indicating value =0p80-07 | Pass/Fail |
| 9 | Verify |  | WSA headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) | Pass/Fail |
| 10 | Verify |  | WSA headerInfo contains expiryTime indicating a Time64 (non-zero value of size 8 bytes | Pass/Fail |
| 11 | Verify |  | WSA headerInfo contains generationLocation indicating latitude (-900000000 . . 900000000) longitude (-1799999999 . . 1800000000) elevation Unit16 | Pass/Fail |

| Identifier |  | TP-16092-WSA-SEND-BV-02 |  |  |
| --- | --- | --- | --- | --- |
| Summary |  | Validate that the IUT will generate a correct 1609.2 WSA certificate data structure. The WSA message shall include signer information, toBesigned data structure and a valid ecdsaP256Signature type. |  |  |
| Test Configuration |  | TC(1) |  |  |
| IUT |  | IUT |  |  |
| Reference: |  |  |  |  |
| PICS Selection |  |  |  |  |
| Pre-test conditions |  |  |  |  |
| - The IUT is being initialized |  |  |  |  |
| Test Sequence |  |  |  |  |
| Step | Type |  | Description | Verdict |
| 1 | Configure |  | The IUT is configured to transmit one or more WSA per second as defined in Table 7-6 |  |
| 2 | Stimulus |  | The IUT transmits WSAs |  |
| 3 | Verify |  | WSA signer contains certificate indicating value=0x01 |  |
| 4 | Verify |  | WSA signer contains version indicating value = 0x03 |  |
| 5 | Verify |  | WSA signer contains type indicating implicit | Pass/Fail |
| 6 | Verify |  | WSA signer contains issuer containing sha256AndDigest indicating HashedId8 (a non-zero value of size 8 octets) | Pass/Fail |
| 7 | Verify |  | WSA toBeSigned contains id indicating none | Pass/Fail |
| 8 | Verify |  | WSA toBeSigned contains cracaId indicating value = 0x0 | Pass/Fail |
| 9 | Verify |  | WSA toBeSigned contains crlSeries indicating value=0x0 | Pass/Fail |
| 10 | Verify |  | WSA WSA toBeSigned contains start indicating Time32 (a non-zero value of size 4 octets) | Pass/Fail |
| 11 | Verify |  | WSA toBeSigned contains duration containing minutes indicating Unit16 (a non-zero value of size 2 bytes) | Pass/Fail |
| 12 | Verify |  | WSA toBeSigned contains region containing circularRegion indicating latitude INTEGER (-900000000..900000000) longitude INTEGER (-1799999999..1800000000) radius INTEGER (0 .. 65535) | Pass/Fail |
| 13 | Verify |  | WSA toBeSigned contains appPermission indicating value=0x01 | Pass/Fail |
| 14 | Verify |  | WSA toBeSigned contains psid indicating value=0p80-07 | Pass/Fail |
| 15 | Verify |  | WSA toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating compressed-y-0 (value of size 32 octets) | Pass/Fail |
| 16 | Verify |  | WSA signature contains ecdsaP256Signature indicating r (a value of compressed-y-0 size of 32 octets) | Pass/Fail |
| 17 | Verify |  | WSA signature contains opaque s indicating non-zero value of size 32 octets | Pass/Fail |

| Identifier |  | TP-16092-WSA-SEND-BV-03 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will generate a well-formed WSA signed by certificate digest of known certificate. The WSA shall include, protocolVersion, content, signedData, tbsData, headerInfo, signer, ecdsaP256Signature. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to transmit one or more WSA per second as defined in Table 7-7 | Pass/Fail |
| 2 | Stimulus | The IUT transmits WSAs | Pass/Fail |
| 4 | Verify | WSA Ieee1609Dot2Data contains content indicating signedData | Pass/Fail |
| 5 | Verify | WSA signedData contains hashId indicating sha256 | Pass/Fail |
| 6 | Verify | WSA tbsData contains protocolVersion indicating value = 0x03 | Pass/Fail |
| 7 | Verify | WSA tbsData contains content indicating unsecuredData (Payload Data> 0) | Pass/Fail |
| 8 | Verify | WSA headerInfo contains psid indicating value =0p80-07 | Pass/Fail |
| 9 | Verify | WSA headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) | Pass/Fail |
| 10 | Verify | WSA headerInfo contains expiryTime indicating a Time64 (non-zero value of size 8 bytes | Pass/Fail |
| 11 | Verify | WSA headerInfo contains generationLocation indicating latitude (-900000000 . . 900000000) longitude (-1799999999 . . 1800000000) elevation Unit16 | Pass/Fail |
| 12 | Verify | WSA contains signer containing digest indicating HashedId8 (a non-zero value of size 8 octets) | Pass/Fail |
| 13 | Verify | WSA signature contains ecdsaP256Signature indicating r (compressed-y-0 consists of octet size 32 ) | Pass/Fail |
| 14 | Verify | WSA signature contains opaque s indicating non-zero value of size 32 octets | Pass/Fail |

| Identifier |  | TP-16092-WSA-SEND-BV-04 |  |  |
| --- | --- | --- | --- | --- |
| Summary |  | Validate that the IUT will generate WSA message digitally signed by certificate that contains a valid signature computed over the entire payload using ecdsaP256Signature type. |  |  |
| Test Configuration |  | TC(1) |  |  |
| IUT |  | IUT |  |  |
| Reference: |  |  |  |  |
| PICS Selection |  |  |  |  |
| Pre-test conditions |  |  |  |  |
| - The IUT is being initialized |  |  |  |  |
| Test Sequence |  |  |  |  |
| Step | Type |  | Description | Verdict |
| 1 | Configure |  | The IUT is configured to transmit one or more WSA per second as defined Table 7-6 |  |
| 2 | Stimulus |  | The IUT transmits WSAs |  |
| 3 | Verify |  | WSA headerInfo contains psid indicating value =0p80-07 | Pass/Fail |
|  | Verify |  | WSA signer contains certificate indicating value=0x01 | Pass/Fail |
| 4 | Verify |  | WSA signer contains version indicating value = 0x03 | Pass/Fail |
| 5 | Verify |  | WSA signer contains type indicating implicit | Pass/Fail |
| 6 | Verify |  | WSA signer contains issuer containing sha256AndDigest indicating ‘CERTID’ | Pass/Fail |
| 7 | Verify |  | WSA toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating (RECVAL) which creates the public key (KEY) by invoking the 1609.2 reconstruction function on (RECVAL) and the public key of the certificate stored on IUT and identified by (CERTID) | Pass/Fail |
| 8 | Verify |  | WSA signature contains ecdsaP256Signature verifiable using (KEY) | Pass/Fail |

#### Reception of packets

| Identifier |  | TP-16092-WSA-RECV-BV-01 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credentials for a well-formed WSA security header. That is, the WSA message shall include protocolVersion, content, signedData, tbsData and headerInfo. |  |
| Test Configuration |  | TC1 |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one WSA per second as defined in Table 7-5 |  |
| 2 | Check | WSA Ieee1609Dot2Data contains protocolVersion indicating (value = 0x03) |  |
| 3 | Check | WSA Ieee1609Dot2Data contains content indicating signedData |  |
| 4 | Check | WSA signedData contains hashId indicating sha256 |  |
| 5 | Check | WSA tbsData contains protocolVersion indicating value = 0x03 |  |
| 6 | Check | WSA tbsData contains content indicating unsecuredData (Payload Data> 0) |  |
| 7 | Check | WSA headerInfo contains psid indicating value =0p80-07 |  |
| 8 | Check | WSA headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) |  |
| 9 | Check | WSA headerInfo contains expiryTime indicating a Time64 (non-zero value of size 8 bytes |  |
| 10 | Check | WSA headerInfo contains generationLocation indicating latitude (-900000000 . . 900000000) longitude (-1799999999 . . 1800000000) elevation Unit16 |  |
| 11 | Stimulate | The IUT receives WSAs |  |
| 12 | Verify | IUT indicates that the WSA message holds a valid security credentials. | Pass/Fail |

| Identifier |  | TP-16092-WSA-RECV-BV-02 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credentials for a well-formed WSA signed by implicit certificate. That is, the certificate data structure shall include signer, toBesigned data structure and ecdsaP256Signature type. |  |
| Test Configuration |  | TC1 |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one WSA per second as defined in Table 7-6. |  |
| 2 | Check | WSA signer contains certificate indicating value=0x01 |  |
| 3 | Check | WSA signer contains version indicating value = 0x03 |  |
| 4 | Check | WSA signer contains type indicating implicit |  |
| 5 | Check | WSA signer contains issuer containing sha256AndDigest indicating HashedId8 a non-zero value of size 8 octets |  |
| 6 | Check | WSA toBeSigned contains id indicating none |  |
| 7 | Check | WSA toBeSigned contains cracaId indicating a value = 0x0 |  |
| 8 | Check | WSA toBeSigned contains crlSeries indicating a value=0x0 |  |
| 9 | Check | WSA WSA toBeSigned contains start indicating Time32 (a non-zero value of size 4 octets) |  |
| 10 | Check | WSA toBeSigned contains duration containing minutes indicating Unit16 (a non-zero value of size 2 bytes) |  |
| 11 | Check | WSA toBeSigned contains region containing circularRegion indicating latitude INTEGER (-900000000..900000000) longitude INTEGER (-1799999999..1800000000) radius INTEGER (0 .. 65535) |  |
| 12 | Check | WSA toBeSigned contains appPermission indicating value=0x01 |  |
| 13 | Check | WSA toBeSigned contains psid indicating a value=0p80-07 |  |
| 14 | Check | WSA toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating compressed-y-0 (value of size 32 octets) |  |
| 15 | Check | WSA signature contains ecdsaP256Signature indicating r (a value of compressed-y-0 size of 32 octets) |  |
| 16 | Check | WSA signature contains opaque s indicating non-zero value of size 32 octets |  |
| 17 | Stimulate | The IUT receives WSAs |  |
| 18 | Verify | IUT indicates that the WSA message holds a valid security credentials. | Pass/Fail |

| Identifier |  | TP-16092-WSA-RECV-BV-03 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate a valid security credentials for a well-formed WSA signed by certificate digest of known certificate. The WSA shall include, protocolVersion, content, signedData, tbsData, headerInfo, signer, ecdsaP256Signature. |  |
| Test Configuration |  | TC(1) |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one WSA per second as defined in Table 7-6. |  |
| 2 | Check | WSA Ieee1609Dot2Data contains protocolVersion indicating value = 0x03 |  |
| 3 | Check | WSA Ieee1609Dot2Data contains content indicating signedData |  |
| 4 | Check | WSA signedData contains hashId indicating sha256 |  |
| 5 | Check | WSA tbsData contains protocolVersion indicating value = 0x03 |  |
| 6 | Check | WSA tbsData contains content indicating unsecuredData (Payload Data> 0) |  |
| 7 | Check | WSA headerInfo contains psid indicating value = 0p80-07 |  |
| 8 | Check | WSA headerInfo contains generationTime indicating a Time64 (non-zero value of size 8 octets) |  |
| 9 | Check | WSA headerInfo contains expiryTime indicating a Time64 (non-zero value of size 8 bytes |  |
| 10 | Check | WSA headerInfo contains generationLocation indicating latitude (-900000000 . . 900000000) longitude (-1799999999 . . 1800000000) elevation Unit16 |  |
| 11 |  | WSA contains signer containing digest indicating HashedId8 (a non-zero value of size 8 octets) |  |
| 12 | Check | WSA signature contains ecdsaP256Signature indicating r (compressed-y-0 consists of octet size 32 ) |  |
| 13 | Check | WSA signature contains opaque s indicating non-zero value of size 32 octets |  |
| 14 | Stimulate | IUT receives WSAs |  |
| 15 | Verify | IUT indicates that the WSA message holds a valid security credentials. | Pass/Fail |

#### Reception of packets – invalid behaviour tests

| Identifier |  | TP-16092-WSA-RECV-BI-01 |  |
| --- | --- | --- | --- |
| Summary |  | Validate that the IUT will indicate an invalid WSA signed by implicit certificate, which failed verification due to incorrect signature. |  |
| Test Configuration |  | TC1 |  |
| IUT |  | IUT |  |
| Reference: |  |  |  |
| PICS Selection |  |  |  |
| Pre-test conditions |  |  |  |
| - The IUT is being initialized |  |  |  |
| Test Sequence |  |  |  |
| Step | Type | Description | Verdict |
| 1 | Configure | The IUT is configured to receive more than one WSA per second |  |
| 3 | Check | WSA headerInfo contains psid indicating value =0p80-07 |  |
|  |  | WSA signer contains certificate indicating value=0x01 |  |
| 4 | Check | WSA signer contains version indicating value = 0x03 |  |
| 5 | Check | WSA signer contains type indicating implicit |  |
|  | Check | WSA signer contains issuer containing sha256AndDigest indicating HashedId8 |  |
| 6 | Check | WSA toBeSigned contains verificationKeyIndicator containing reconstructionValue indicating ‘RECVAL’ which creates the public key ‘KEY’ by invoking the 1609.2 reconstruction function on ‘RECVAL’ and the public key of the certificate stored on IUT and identified by ‘DG1’ |  |
| 7 | Check | WSA signature contains ecdsaP256Signature indicating r and s not verifiable using (KEY) |  |
| 8 | Stimulate | The IUT receives the WSAs |  |
| 8 | Verify | IUT indicates that the WSA message holds an invalid security credentials. | Pass/Fail |

# Messages and information element content

This section contains basic message structure that will be used in the TP’s.

## Basic Safety message (BSM):

### Message defaults

The following assumptions apply to all messages defined in this section.

- All default values are listed in section 4.1

- The ASN.1 presentation in this section depicts the 1609.2 [8] secure message formats structure of WSM message.

### Secure PDU Message Details

- Table 7-1 describes 1609.2[8] security header information of BSM which includes the payload.

- Table 7-2 and Table 7-3 describes 1609.2[8] signer credentials information of BSM.

- Table 7-4 describes 1609.2[8] security signature information of BSM.

#### BSM Header information

Table 7-1: BSM Security Header Information

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Ieee1609Dot2Data SEQUENCE { |  |  |
| protocolVersion | 3 |  |
| content signedData SEQUENCE { |  |  |
| hashId | sha256 |  |
| tbsData SEQUENCE{ |  |  |
| payload SEQUENCE { |  |  |
| data { |  |  |
| protocolVersion | 3 |  |
| content | Any valid BSM payload including 1609.3 WAVE message information. | BSM payload created according to 2945/1 and 2735 standards |
| } |  |  |
| } |  |  |
| headerInfo { |  |  |
| psid | 32 (PSID=0p20) | PSID value for BSM is 0p20 |
| generationTime | Any valid value |  |
| } |  |  |
| } |  |  |
| Require signer credentials information in Table 7-2 or Table 7-3 |  |  |
| Require Security Signature information in Table 7-4 |  |  |

#### BSM Signed with Certificate Digest

Table 7-2: BSM Signed by Signer type of Certificate Digest

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Requires BSM Security header information in Table 7-1 |  |  |
| signer { } | digest | HashedID8 |
| Require Security Signature information in Table 7-4 |  |  |

#### BSM Signed with Implicit Certificate

Table 7-3: BSM Signed by Signer type of Implicit Certificate

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Requires BSM Security header information in Table 7-1 |  |  |
| signer SEQUENCE { | certificate |  |
| certificate { | 1 |  |
| version | 3 |  |
| type | implicit |  |
| issuer | ecdsaNistP256AndDigest | HashedID8 |
| toBeSigned SEQUENCE{ |  |  |
| id { | linkageData |  |
| iCert | Any valid value |  |
| linkage-value | Any Valid value |  |
| group-linkage-value SEQUENCE{ |  |  |
| jValue | Any valid value |  |
| value | Any valid value |  |
| } |  |  |
| } |  |  |
| cracaId | Any valid value |  |
| crlSeries | 1 |  |
| validityPeriod SEQUENCE{ |  |  |
| start | Any valid value |  |
| duration hours | Any valid value |  |
| } |  |  |
| region identifiedRegion SEQUENCE { | 3 |  |
| countryOnly | 124 |  |
| countryOnly | 484 |  |
| countryOnly | 840 |  |
| }, |  |  |
| appPermissions SEQUENCE{ | 2 |  |
| { |  |  |
| psid | 32 (PSID=0p20) | BSM |
| } |  |  |
| { |  |  |
| psid | 38 (PSID=0p26) | Misbehaviour for common applications |
| } |  |  |
| } |  |  |
| verifyKeyIndicator | reconstructionValue | compressed-y-0 |
| } |  |  |
| } |  |  |
| } |  |  |
| Require Security Signature information in Table 7-4 |  |  |

#### BSM security signature

Table 7-4: BSM Security Signature

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Requires BSM Security header information in Table 7-1 |  |  |
| Require signer credentials information in Table 7-2 or Table 7-3 |  |  |
| signature SEQUENCE { | ecdsa256Signature | EccP256CurvePoint |
| r | compressed-y-0 | Octet size of 32 |
| s |  | Octet size of 32 |
| } |  |  |

### WSA Message Details

- Table 7-5 describes 1609.2[8] security header information of WSA which includes valid payload.

- Table 7-6 and Table 7-7 describes 1609.2[8] signer credentials information of WSA.

- Table 7-8 describes 1609.2[8] security signature information of WSA.

#### WSA Header information

Table 7-5: WSA Security Header Information

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Ieee1609Dot2Data SEQUENCE { |  |  |
| protocolVersion | 3 |  |
| content signedData SEQUENCE { |  |  |
| hashId | sha256 |  |
| tbsData SEQUENCE { |  |  |
| payload SEQUENCE { |  |  |
| data { |  |  |
| protocolVersion | 3 |  |
| content | Valid WSA payload |  |
| } |  |  |
| } |  |  |
| headerInfo SEQUENCE{ |  |  |
| psid | 135 (PSID=0p8007) |  |
| generationTime | Any valid value |  |
| expiryTime | Any valid value |  |
| generationLocation SEQUENCE { |  |  |
| latitude | Any valid value |  |
| longitude | Any valid value |  |
| elevation | Any valid value |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Require signer credentials information in Table 7-6 or 7-7 |  |  |
| Require Security Signature information in Table 7-8 |  |  |
| } |  |  |

#### WSA Signed with Implicit Certificate

Table 7-6: WSA Signed by Signer type of Implicit Certificate

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Requires WSM Security header information in Table 7-5 |  |  |
| signer SEQUENCE { |  |  |
| certificate { | 1 |  |
| Version | 3 |  |
| type | implicit |  |
| issuer | ecdsaNistP256AndDigest | HashedID8 |
| toBeSigned SEQUENCE { |  |  |
| id | none |  |
| cracaId | Value = 0 |  |
| crlSeries | Value=0 |  |
| validityPeriod SEQUENCE { |  |  |
| start | Any valid value |  |
| duration minutes | Any valid value |  |
| } |  |  |
| region circularRegion SEQUENCE { |  |  |
| centre { |  |  |
| latitude | Any valid value |  |
| longitude | Any valid value |  |
| } |  |  |
| radius | Any valid value |  |
| } |  |  |
| appPermissions { |  |  |
| { |  |  |
| psid | 135 (PSID=0p8007) |  |
| } |  |  |
| } |  |  |
| verifyKeyIndicator | reconstructionValue | compressed-y-0 |
| } |  |  |
| } |  |  |
| } |  |  |
| Require Security Signature information in Table 7-8 |  |  |

#### WSA Signed with signer type of Certificate digest

Table 7-7: WSA Signed with Certificate digest

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Requires WSA Security header information in Table 7-5 |  |  |
| signer { } | digest | HashedID8 |
| Require Security Signature information in Table 7 8 |  |  |

#### WSA security signature

Table 7-8: WSA Security Signature

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| Requires WSM Security header information in Table 7-5 |  |  |
| Require signer credentials information in Table 7-6 or Table 7-7 |  |  |
| signature SEQUENCE { | ecdsa256Signature | EccP256CurvePoint |
| r | compressed-y-0 | Octet size of 32 |
| s | Any valid value | Octet size of 32 |
| } |  |  |

Traceability Matrix

This section of the document contains the traceability matrix for BSM and WSA. As shown below, Table A- 1 lists BSM IEEE 1609.2[8] traceability to TPs. In Page (# 40) Table A- 2 lists WSA IEEE 1609.2 traceability to TPs where PICS for WSA was derived from “IEEE 1609.2[8] security specification for WSA requirements” listed under Annex H in 1609.3[5].

The current test specification doesn’t include any TP’s that requires Security Credential Management System (SCMS) due to the fact that the new standard is not available and will be available in 2016. Accordingly, not all the mandatory requirements by 2945/1 is tested at this time.

Table A- 1: BSM IEEE 1609.2 PICS traceability to TPs

| 1609.2 PICS from [8] | Features in [8] | Reference section in [8] | Status (J2945-1 [1]) | Support (J2945-1 [1]) | TP ID | TP Description |
| --- | --- | --- | --- | --- | --- | --- |
| S1.2.2 | Create Ieee1609Dot2Data containing valid SignedData | 4.2.2.2.3, 5.2, 5.3.1, 5.3.3,5.3.7,6.3.4,6.3.9,9.3.9.1 | S1.2:O3 | Y | TP-16092-BSM-SEND-BV-01 | To verify that the IUT will generate a valid signedData as per 1609.2[8] specifications |
| S1.2.2.1 | Using a valid HashAlgorithm | 6.3.5 | S1.2.2:M | Y | TP-16092-BSM-SEND-BV-01 | To verify that the IUT will generate a valid signedData using sha256 hash |
|  |  |  |  |  | TP-16092-BSM-SEND-BV-03 | To verify that the IUT will generate signed using certificate digest generated by hash 256 |
| S1.2.2.1.1 | Support signing with hash algorithm SHA-256 | 6.3.5 | S1.2.2:M | Y | Refer to S1.2.2.1 |  |
| S1.2.2.2 | Containing a Signed Data payload | 6.3.6 | S1.2.2:M | Y | TP-16092-BSM-SEND-BV-01 | To verify that the IUT will generate a signedData with BSM payload is included |
| S1.2.2.2.1 | with payload containing data | 6.3.7 | S1.2.2.2:O4 | Y | Refer to S1.2.2.2 |  |
| S1.2.2.2.3. | … with generationTime in the security header | 6.3.9, 6.3.11 | S1.2.2.2: O | Y | TP-16092-BSM-SEND-BV-01 | To verify that the IUT will generate BSM security header that includes generationTime |
|  |  |  |  |  | TP-16092-BSM-SEND-BV-03 | To verify that the IUT will generate BSM security header that includes generationTime signed by certificate digest |
| S1.2.2.3. | Support a SignerIdentifier | 6.3.24 | S1.2.2:M | Y | TP-16092-BSM-SEND-BV-02 | To verify that the IUT will generate aBSM signed with signer type of certificate |
|  |  |  |  |  | TP-16092-BSM-SEND-BV-03 | To verify that the IUT will generate BSM signed with signer type of certificate digest |
| S1.2.2.3.1. | … of type digest | 6.3.26 | S1.2.2.3:O6 | Y | Refer to S1.2.2.3 |  |
| S1.2.2.3.2. | … of type certificate | 6.4.2 | S1.2.2.3:O6 | Y | Refer to S1.2.2.3 |  |
| S1.2.2.3.2.1 | Maximum number of Certificates in the chain | 5.1.2.2 | S1.2.2.3.2 8:M > 8:O | 1 | TP-16092-BSM-SEND-BV-02 | To verify that the IUT will generate BSM signed with signer type of certificate With a Maximum number of certificates in the chain is equal to 1. |
| S1.2.2.4. | Support a Signature | 6.3.28 | S1.2.2:M | Y | TP-16092-BSM-SEND-BV-05 | To verify that the IUT will generate a valid signature to sign BSM message generated by signer of type certificate digest |
|  |  |  |  |  | TP-16092-BSM-SEND-BV-06 | To verify that the IUT will generate a valid signature to sign BSM messages generated by signer of type certificate |
| S1.2.2.4.1. | … a ecdsa256Signature | 6.3.29 | S1.2.2.4:M | Y | TP-16092-BSM-SEND-BV-02 | To verify that the IUT will generate a valid signature using ecdsa256Signature type. Where it uses the NIST p256 algorithm to generate the compressed r value. The signer type used to sign the BSM message is certificate |
|  |  |  |  |  | TP-16092-BSM-SEND-BV-03 | To verify that the IUT will generate a valid signature using ecdsa256Signature type. Where it uses the NIST p256 algorithm to generate the compressed r value. The signer type used to sign the BSM message is digest |
| S1.2.2.4.1.1. | … … using NIST p256 | 6.3.29 | S1.2.2.4.1:O7 | Y | Refer to S1.2.2.4.1 |  |
| S1.2.2.4.1.4. | … … with a compressed r value | 6.3.23 | S1.2.2.4.1:O8 | Y | Refer to S1.2.2.4.1 |  |
| S1.2.2.5.1. | Determine that the region is correct | 6.4.8, 6.4.17 | S1.2.2.5:O | Y | TP-16092-BSM-SEND-BV-02 | To verify that the certificate region is defined as " identifiedRegion" with a minimum number of 3 countries as specified in SAE J2945/1 |
| S1.2.2.5.1.4 | Support identifiedRegion | 6.4.17, 6.4.22 | S1.2.2.5.1:O9 | Y | Refer to S1.2.2.5.1 |  |
| S1.2.2.5.1.4.1. | Maximum number of identifiedRegions supported | 6.4.17 6.4.22 | S1.2.2.5.1.4: 8:M >8:O | Minimum of 3 Note: US, Canada, Mexico supported as defined by the United Nations Statistics Division, October 2013 edition | Refer to S1.2.2.5.1 |  |
| S1.2.2.5.1.4.2. | Support IdentifiedRegion of type Country Only | 6.4.22, 6.4.23 | S1.2.2.5.1.4:O1 | Y | Refer to S1.2.2.5.1 |  |
| S1.2.2.5.2 | Determine that the certificate has the proper appPermissions | 6.4.8 6.4.28 | S1.2.2.5:O | y | TP-16092-BSM-SEND-BV-02 | verify that the IUT will generate a signedData using implicit certificate that contains the appropriate appPermissions |
| S1.2.2.8. | Support signing with implicit certificate | 5.3.2, 6.4.5 | S1.2.2.5:O11 | Y | Refer to S1.2.2.5.2 |  |
| S1.3.2. | Verify Ieee1609Dot2Data containing SignedData | 4.2.2.2.3, 5.2, 5.3.1, 5.3.3 5.3.7, 6.3.4,6.3.9 | S1.3:O17 | Y | TP-16092-BSM-RECV-BV-01 | To verify that the IUT will accept a valid BSM contains signedData. |
| S1.3.2.1. | Using a valid HashAlgorithm |  | S1.3.2:M | Y | TP-16092-BSM-RECV-BV-01 | To verify that the IUT will accept BSM message signed by a digest of type sha256 |
|  |  |  |  |  | TP-16092-BSM-RECV-BV-03 | To verify that the IUT will accept BSM messages signed by a signer credential of type certificate digest using sha256 |
| S1.3.2.1.1. | Verify signed data using Hash Algorithm SHA-256 | 6.3.5 | S1.3.2.1:M | Y | Refer to S1.3.2.1 |  |
| S1.3.2.2. | Containing a Signed Data payload | 6.3.6 | S1.3.2:M | y | Refer to S1.3.2 |  |
| S1.3.2.2.1. | … with payload containing data | 6.3.7 | S1.3.2.2:O18 | Y | Refer to S1.3.2 |  |
| S1.3.2.2.3. | … with generation Time in the security header | 6.3.9, 6.3.11 | S1.3.2.2:O | Y | TP-16092-BSM-RECV-BV-01 | To verify that the IUT will accept BSM message with the correct security header information. That is, it must contain generationTime. |
| S1.3.2.3. | Support a SignerIdentifier | 6.3.24 | S1.3.2:M | Y | TP-16092-BSM-RECV-BV-02 | To verify that the IUT will accept BSM message signed with the correct signer credential of type certificate. . |
|  |  |  |  |  | TP-16092-BSM-RECV-BV-03 | To verify that the IUT will accept BSM message signed with the correct signer credential of type certificate digest. |
| S1.3.2.3.1. | … of type digest | 6.3.26 | S1.3.2.3:O20 | Y | Refer to S1.3.2.3 |  |
| S1.3.2.3.2 | … of type certificate | 6.4.2 | S1.3.2.3:O20 | Y | Refer to S1.3.2.3 |  |
| S1.3.2.3.2.1. | … … Maximum number of Certificates in the chain | 5.1.2.2 | S1.3.2.3.2 1:M >1:O | 1 | TP-16092-BSM-RECV-BV-02 | To verify that the IUT will accept a BSM message with a maximum certificate chain is equal to 1. |
| S1.3.2.4. | Support a Signature | 6.3.28 | S1.3.2:M | Y | TP-16092-BSM-RECV-BV-02 | To verify that the IUT will accept BSM message signed by ecdsa256Signature type. Where it uses the NIST p256 algorithm to generate the compressed r value. The signer credential type used to sign the BSM message is certificate |
|  |  |  |  |  | TP-16092-BSM-RECV-BV-03 | To verify that the IUT will accept BSM message signed by ecdsa256Signature type. Where it uses the NIST p256 algorithm to generate the compressed r value. The signer credential type used to sign the BSM message is certificate digest. |
| S1.3.2.4.1. | … a ecdsa256Signature | 6.3.29 | S1.3.2.4:M | Y | Refer to S1.3.2.4 |  |
| S1.3.2.4.1.1. | … … using NIST p256 | 6.3.29 | S1.3.2.4.1:O21 | Y | Refer to S1.3.2.4 |  |
| S1.3.2.4.1.4. | … … with a compressed r value | 6.3.23 | S1.3.2.4.1:O22 | Y | Refer to S1.3.2.4 |  |
| S1.3.2.10.14. | … SPDU-Crypto: Verification failure | 5.3.1 | S1.3.2.10:M | Y | TP-16092-BSM-RECV-BI-01 | To verify that the IUT will reject a BSM message signed with invalid ecdsa256Signature. The signer credential of type certificate digest is used to sign the BSM message. |
|  |  |  |  |  | TP-16092-BSM-RECV-BI-02 | To verify that the IUT will reject a BSM message signed with invalid ecdsa256Signature. The signer credential of type certificate is used to sign the BSM message. |

Table A- 2: WSA IEEE 1609.2 PICS traceability to TPs

| 1609.2 PICS from [8] | Features in [8] | Reference section in [8] | Status [8] | Support 1609.3[5] | TP ID | TP Description |
| --- | --- | --- | --- | --- | --- | --- |
| S1.2.2 | Create Ieee1609Dot2Data containing valid SignedData | 4.2.2.2.3, 4.2.2.2.3, 5.2, 5.3.1 5.3.3, 5.3.7, 6.3.4, 6.3.9, 9.3.9.1 | S1.2:O3 | Y | TP-16092-WSA-SEND-BV-01 | To verify that the IUT will generate a valid WSA signedData as per 1609.2[8]specifications |
| S1.2.2.1. | Using a valid HashAlgorithm | 6.3.5 | S1.2.2:M | Y | TP-16092-WSA-SEND-BV-01 | To verify that the IUT will generate a valid WSA signedData using sha256 hash |
| S1.2.2.1.1. | Support signing with hash algorithm sha-256 | 6.3.5 | S1.2.2:M | Y | Refer to S1.2.2.1 |  |
| S1.2.2.2. | Containing a Signed Data payload | 6.3.6 | S1.2.2:M | Y | TP-16092-WSA-SEND-BV-01 | To verify that the IUT will generate a valid signedData with WSA payload is included |
| S1.2.2.2.1. | … with payload containing data | 6.3.7 | S1.2.2.2:O4 | Y | Refer to S1.2.2.2 |  |
| S1.2.2.2.3. | … with generationTime in the security headers | 6.3.9, 6.3.11 | S1.2.2.2: O | Y | TP-16092-WSA-SEND-BV-01 | To verify that the IUT will generate a valid WSA headerinfo data structure that include Generation time |
| S1.2.2.2.4. | … with expiryTime in the security headers | 6.3.9, 6.3.11 | S1.2.2.2: O | Y | TP-16092-WSA-SEND-BV-01 | To verify that the IUT will generate a valid WSA headerinfo data structure that includes Expiry Time. |
| S1.2.2.2.5. | … with generationLocation in the security headers | 6.3.9, 6.3.12 | S1.2.2.2: O | Y | TP-16092-WSA-SEND-BV-01 | To verify that the IUT will generate a valid WSA headerinfo data structure that include Generation location. |
| S1.2.2.3. | Support a SignerIdentifier | 6.3.24 | S1.2.2:M | Y | TP-16092-WSA-SEND-BV-02 | To verify that the IUT will generate WSA signed with signer type of implicit certificate |
|  |  |  |  |  | TP-16092-WSA-SEND-BV-03 | To verify that the IUT will generate WSA signed with signer type of certificate digest |
| S1.2.2.3.1. | … of type digest | 6.3.26 | S1.2.2.3:O6 | Y | Refer to S1.2.2.3 |  |
| S1.2.2.3.2. | … of type certificate | 6.4.2 | S1.2.2.3:O6 | Y | Refer to S1.2.2.3 |  |
| S1.2.2.3.2.1. | … … Maximum number of Certificates in the chain | 5.1.2.2 | S1.2.2.3.2 8:M >8O | 1 | TP-16092-WSA-SEND-BV-02 | To verify that the IUT Will generate WSA signed with certificate chain =1 |
| S1.2.2.4. | Support a Signature | 6.3.28 | S1.2.2:M | Y | TP-16092-WSA-SEND-BV-04 | To verify that the IUT Will generate WSA signed with a valid signature. The signature will be generated using NISTp256 and using Compressed r value |
| S1.2.2.4.1. | … a ecdsa256Signature | 6.3.29 | S1.2.2.4:M | Y | Refer to S1.2.2.4. |  |
| S1.2.2.4.1.1. | … … using NIST p256 | 6.3.29 | S1.2.2.4.1:O7 | Y | Refer to S1.2.2.4. |  |
| S1.2.2.4.1.4. | … … with a compressed r value | 6.3.23 | S1.2.2.4.1:O8 | Y | Refer to S1.2.2.4. |  |
| S1.2.2.5.1. | Determine that the region is correct | 6.4.8, 6.4.17 | S1.2.2.5:O | Y | TP-16092-WSA-SEND-BV-02 | To verify that the IUT will generated a signer of type implicit certificate that contains a valid region. |
| S1.2.2.8. | Support signing with implicit certificates | 5.3.2, 6.4.5 | S1.2.2.5:O11 | Y | Refer to S1.2.2.3 |  |
| S1.3.2. | Verify Ieee1609Dot2Data containing SignedData | 4.2.2.2.3, 5.2, 5.3.1 5.3.3, 5.3.7, 6.3.4 6.3.9 | S1.3:O17 | Y | TP-16092-WSA-RECV-BV-01 | To verify that the IUT will accept a valid WSA contains signedData. |
| S1.3.2.1. | Using a valid HashAlgorithm |  | S1.3.2:M | Y | TP-16092-WSA-RECV-BV-01 | To verify that the IUT will accept WSA message signed by a digest of type sha256 |
|  |  |  |  |  | TP-16092-WSA-RECV-BV-03 | To verify that the IUT will accept BSM messages signed by a signer credential of type certificate digest using sha256 |
| S1.3.2.1.1. | Verify signed data using HashAlgorith SHA-256 | 6.3.5 | S1.3.2.1:M | Y | Refer to S1.3.2.1 |  |
| S1.3.2.2. | Containing a Signed Data payload | 6.3.6 | S1.3.2:M | Y | TP-16092-WSA-RECV-BV-01 | To verify that the IUT will accept a WSA signed message containing Payload |
| S1.3.2.2.1. | … with payload containing data | 6.3.7 | S1.3.2.2:O18 | Y | Refer to S1.3.2.2 |  |
| S1.3.2.2.3. | … with generationTime in the security headers | 6.3.9, 6.3.11 | S1.3.2.2:O | Y | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a valid WSA headerinfo data structure that include Generation time |
| S1.3.2.2.4. | … with expiryTime in the security headers | 6.3.9, 6.3.11 | S1.3.2.2:O | Y | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a valid WSA headerinfo data structure that include Expiry time. |
| S1.3.2.2.5. | … with generationLocation in the security headers | 6.3.9, 6.3.12 | S1.3.2.2:O | Y | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a valid WSA headerinfo data structure that include Generation location |
| S1.3.2.3. | Support a SignerIdentifier | 6.3.24 | S1.3.2:M | Y | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a valid WSA message signed with signer type of implicit certificate |
|  |  |  |  |  | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a valid WSA message signed with signer type of certificate digest. |
| S1.3.2.3.1. | … of type digest | 6.3.26 | S1.3.2.3:O20 | Y | Refer to S1.3.2.3 |  |
| S1.3.2.3.2. | … of type certificate | 6.4.2 | S1.3.2.3:O20 | Y | Refer to S1.3.2.3. |  |
| S1.3.2.3.2.1. | … … Maximum number of Certificates in the chain | 5.1.2.2 | S1.3.2.3.2 1:M >1:O | 1 | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a valid WSA message signed with certificate chain = 1 |
| S1.3.2.4. | Support a Signature | 6.3.28 | S1.3.2:M | Y | TP-16092-WSA-RECV-BV-02 | To verify that the IUT Will accept WSA signed with a valid signature. The signature will be generated using NISTp256 and using Compressed r value |
| S1.3.2.4.1. | … a ecdsa256Signature | 6.3.29 | S1.3.2.4:M | Y | Refer to S1.3.2.4 |  |
| S1.3.2.4.1.1. | … … using NIST p256 | 6.3.29 | S1.3.2.4.1:O21 | Y | Refer to S1.3.2.4 |  |
| S1.3.2.4.1.4. | … … with a compressed r value | 6.3.23 | S1.3.2.4.1:O22 | Y | Refer to S1.3.2.4 |  |
| S1.3.2.5.1.1. | … using a circularRegion | 6.4.17, 6.4.18 | S1.3.2.5.1:O23 | Y | TP-16092-WSA-RECV-BV-02 | To verify that the IUT will accept a WSA message signed by a signer of type implicit certificate with a region of type circular. |
| S1.3.2.7. | Support verifying SPDUs signed with implicit authorization certificates | 5.3.2, 6.4.5 | S1.3.2:O25 | Y | Refer to S1.3.2.3. |  |
| S1.3.2.10.14. | … SPDU-Crypto: Verification failure | 5.3.1 | S1.3.2.10:M | Y | TP-16092-WSA-RECV-BI-01 | To verify that the IUT will reject a WSA message signed with invalid ecdsa256Signature. The signer credential of type certificate is used to sign the BSM message. |

# Revision History

| V0.1.0 | Sep 17, 2015 | Initial Draft – BSM test cases |
| --- | --- | --- |
| V0.2.0 | Sep 30, 2015 | Added test cases for WSA messages |
| V0.3.0 | Oct 5, 2015 | Updated BSM and WSA messages |
| V.0.4.0 | Oct 23, 2015 | Updated Test Cases to the new format |
| V.0.5.0 | Dec 31, 2015 | Updated TP to the new Standard Added Tractability Matrix for BSM and WSA |
| V.0.6.0 | Feb 5, 2016 | Based on peer review, multiple changes were made to the document. |
| V.1.0 | March 23, 2016 | Incorporated comments from industry reviewers |

◙ End of Document ◙

BSMs will have certificates with a lifetime of a week and will be revocable. cracaId will be non-zero, crlSeries value will be 1 and linkageData is used to determine if the cert is revoked. reconstructionValue and r values will use compressed-y-0  for elliptic curve point is encoding.

All WSA test cases are written with the assumption that the signer credentials (certificate) are non-revocable, because they will have short lifetimes. Certificate geographical region will be circularRegion type reconstructionValue and r values will use compressed-y-0 for elliptic curve point encoding.